SafeGrowth Vault

Architecture Decision Record

ADR-002

Pebble over RocksDB for Local Storage

Pure-Go embedded key-value store for shards, manifest, and credit ledger

| ADR ID | ADR-002 |
| --- | --- |
| Status | Accepted |
| Date | March 2026 |
| Deciders | Glenn + remote collaborators |
| SRD References | ST-01, ST-03, ST-06, ST-07, NF-09, NF-15 |
| Related ADRs | ADR-001 (zero CGO — primary driver) |

# 1. Context

The node daemon requires a local embedded key-value store for three distinct purposes:

- Shard storage — opaque binary blobs keyed by SHA3-256 hash (shards/ key prefix)

- Shard manifest — metadata: shard ID, size, owner pubkey, placement timestamp, last verified timestamp (manifest/ key prefix)

- Credit ledger — append-only log of signed credit entries (credits/ key prefix)

The initial design specified RocksDB via the gorocksdb Go binding. RocksDB is the industry standard for embedded key-value storage in high-performance distributed systems — used in production by CockroachDB, TiKV, and many others. However, gorocksdb is a CGO binding, violating ADR-001. Additionally, RocksDB's memory footprint on constrained hardware (Raspberry Pi 3B: 1GB RAM, NF-15) is a documented concern.

# 2. Decision

Use github.com/cockroachdb/pebble as the embedded key-value store for all three data namespaces.

Pebble is CockroachDB's purpose-built replacement for RocksDB. Key properties:

- Pure Go — zero CGO, compiles with CGO_ENABLED=0 on all targets

- WAL-backed for crash safety (satisfies NF-09: daemon must recover cleanly from crash without manual intervention)

- Lower memory footprint than RocksDB at equivalent workloads — suits Raspberry Pi 3B (NF-15)

- Actively maintained by CockroachDB's production engineering team

- API-compatible with RocksDB at the conceptual level (similar iterator/batch/compaction model)

The three namespaces (shards/, manifest/, credits/) are separated by key prefixes within a single Pebble instance, reducing file handle usage and allowing independent iteration and backup per namespace.

NAS v1.2 UPDATE: Pebble operates correctly on ZFS and Btrfs COW filesystems used by NAS devices (Synology DSM, TrueNAS). Double-journaling (Pebble WAL + ZFS/Btrfs journal) is documented and accepted. On NAS devices with battery-backed write cache (BBU), write-through cache mode must be configured on the SafeGrowth data volume to preserve fsync() durability semantics — Pebble cannot detect BBU at the application layer. NAS data directories must be excluded from automated snapshots to prevent inconsistent Pebble WAL state on snapshot restore.

# 3. Consequences

## 3.1 Positive

- Satisfies zero-CGO constraint (ADR-001) — no C toolchain required

- WAL guarantees crash safety without additional work (satisfies NF-09, ST-06)

- Lower memory footprint suits Raspberry Pi 3B target (NF-15)

- Single Pebble instance handles all three data namespaces, reducing file handle usage

- RocksDB-compatible mental model means team documentation and debugging intuition transfers

## 3.2 Negative / Trade-offs

- Pebble is less widely deployed outside of CockroachDB than RocksDB

- Some RocksDB-specific tuning guides don't apply directly to Pebble

- Pebble's compaction behaviour at very large shard stores (>1TB) is less documented than RocksDB's

## 3.3 Neutral

- Both RocksDB and Pebble use LSM-tree architecture — performance characteristics are similar for shard storage patterns (mostly write-once, read-occasionally)

- Key-prefix namespace separation is a Pebble convention; it would have been needed with RocksDB as well

# 4. Alternatives Considered

| Alternative | Rejection Reason |
| --- | --- |
| RocksDB via gorocksdb | CGO dependency violates ADR-001. |
| bbolt (BoltDB) | Uses a B-tree (not LSM) — performs poorly for write-heavy workloads at shard sizes we expect (individual shards can be hundreds of MB). Also holds entire database file mmap'd, problematic on memory-constrained devices. |
| SQLite via modernc/sqlite (pure Go CGO-free port) | SQL semantics add unnecessary complexity for a key-value workload. The transpiled C-to-Go code is less auditable than a native Go implementation. |
| Custom flat-file storage | Implementing crash-safe atomic writes, compaction, and range iteration correctly is a significant engineering investment with high implementation risk. |

# 5. Related ADRs

- ADR-001 (Zero CGO Constraint) — the primary driver of this decision

- ADR-011 (Super Anchor / NAS) — adds ZFS/Btrfs COW filesystem notes and BBU write cache guidance